# CYCLIC EXECUTIVE DUAL LED EMBEDDED SYSTEM

Document name: Cyclic Executive Dual LED Embedded System

Document reference: cmpe310.fall25.project5

Revision: 1.0 — ECN revision of cmpe310.fall25.project4

Date of publication: [Your Date Here]

LEAD ENGINEER: Oluwadamilola Kuforiji, University of Maryland Baltimore County

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA

MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

# DESCRIPTION

The system performs identically to PROJECT-ASYNC: it controls two LEDs on Digital Pins 2 and 3 of an Arduino Uno R3, allowing a user to independently set each LED’s blink interval in milliseconds via the Arduino IDE serial monitor. The design change mandated by ECR 0.1 replaces the ad-hoc cooperative multitasking structure of PROJECT-ASYNC with a formal round-robin Cyclic Executive task manager. Tasks are registered in a function pointer array and dispatched in sequence on each pass of the main loop. This architecture improves reliability and makes it straightforward to add future tasks without restructuring the scheduler.

# ENGINEERING CHANGE REQUEST

ECR Number: ECR 0.1

Date: January 1, 2030

Requestor: Prof. Kidd

Originating Document: cmpe310.fall25.project4

Description of Change: Modify the design resulting from the high-level requirements defined in the project 2 assignment to use a round-robin cyclic executive task manager. There is no change to the Customer Requirements or the High-Level Technical Requirements defined in the project 2 guidelines.

Reason for Change: Results in a more reliable design. The new design will reduce labor and design costs for implementing future extensions.

# ENGINEERING CHANGE NOTE (ECN)

The following document has been revised in response to ECR 0.1:

Previous document: Project 1 ASYNC

This document: Kuforiji.Oluwadamilola.cmpe310.fall25.project5

## Summary of Changes

- SOFTWARE ARCHITECTURE (Design section): Replaced ad-hoc cooperative multitasking with a formal round-robin Cyclic Executive task manager dispatched via a function pointer array.

- SOURCE CODE (Design section): Rewritten to implement the Cyclic Executive. All task logic (serial input, LED 1 blink, LED 2 blink) are in dedicated functions registered in the task array using the declaration: void (*tasks[])() = {getSerialInput, blinkPin2, blinkPin3};

- TESTING AND VALIDATION REQUIREMENTS: Added T.9 and T.10 to verify correct cyclic executive dispatch behavior. All prior tests T.1–T.8 are retained unchanged.

- DEFINITIONS AND ABBREVIATIONS: Added definitions for Cyclic Executive, ECR, ECN, Function Pointer, Round-Robin, and Task.

No change to: Customer Requirements, High-Level Technical Requirements, Educational Constraints, Hardware Design, or Conventions.

# REFERENCES

1. cmpe310.fall25.project4 — Asynchronous Dual LED Embedded System (PROJECT-ASYNC baseline document)

2. Arduino Reference – millis(): https://www.arduino.cc/reference/en/language/functions/time/millis/

3. ATmega328P Datasheet – Microchip Technology

4. Arduino IDE 2.3.3 Documentation: https://docs.arduino.cc/

# DEFINITIONS AND ABBREVIATIONS

ATmega328P – 8-bit AVR microcontroller used on the Arduino Uno R3

Cyclic Executive – A simple scheduler that repeatedly calls a fixed set of tasks in round-robin order

ECR – Engineering Change Request

ECN – Engineering Change Note; documents the changes made in response to an ECR

Function Pointer – A variable that stores the address of a function, enabling it to be called indirectly

IDE – Integrated Development Environment

LED – Light Emitting Diode

millis() – Arduino function returning elapsed time in milliseconds since program start

Non-blocking – Code that does not halt program execution while waiting

PROJECT-ASYNC – The previous project upon which this project is based; Ref: Kuforiji.Oluwadamilola.cmpe310.fall25.project4

Round-Robin – A scheduling scheme in which tasks are dispatched in a fixed cyclic order

Serial Monitor – Terminal interface in the Arduino IDE for serial communication

Task – A discrete unit of work (function) registered in the cyclic executive task array

# CONVENTIONS

Must, shall, or will – The design must satisfy the requirement.

May – The design may satisfy the requirement but does not have to.

Informative – The intent of the following description is to make the requirement more understandable.

All customer requirements are prefixed with C.#

All high-level requirements are prefixed with HL.#

All testing/validation requirements are prefixed with T.#

All educational constraints are prefixed with E.#

# CUSTOMER REQUIREMENTS

The customer requirements C.1 through C.3 (including C.1.1 and C.2.1) are identical to those of PROJECT-ASYNC.

Ref: cmpe310.fall25.project4, CUSTOMER REQUIREMENTS section.

# HIGH-LEVEL TECHNICAL REQUIREMENTS

The high-level technical requirements HL.1 through HL.6 are identical to those of PROJECT-ASYNC.

Ref cmpe310.fall25.project4, HIGH-LEVEL TECHNICAL REQUIREMENTS section.

The Educational Constraints E.1 through E.4 are identical to those of PROJECT-ASYNC.

Ref: cmpe310.fall25.project4, HIGH-LEVEL TECHNICAL REQUIREMENTS section (Educational Constraints).

# TESTING AND VALIDATION REQUIREMENTS

The testing and validation requirements T.1 through T.8 are identical to those of PROJECT-ASYNC.

Ref: Kuforiji.Oluwadamilola.cmpe310.fall25.project4, TESTING AND VALIDATION REQUIREMENTS section.

Additional requirements added by this revision (ECR 0.1):

T.9  Verify that all tasks (getSerialInput, blinkPin2, blinkPin3) are registered in the cyclic executive function pointer array.

T.10  Verify that the cyclic executive dispatches all tasks in round-robin order on each iteration of the main loop, with no task skipped or blocking another.

# DESIGN

## Hardware Design

The hardware design is identical to that of PROJECT-ASYNC.

Ref: Kuforiji.Oluwadamilola.cmpe310.fall25.project4, DESIGN — Hardware Design section.

Components:

- Arduino Uno R3 (ATmega328P)

- 2 LEDs

- 2 x 200Ω resistors

- USB cable for programming and power

Connection Details:

LED1 → Digital Pin 2 → 200Ω resistor → Ground

LED2 → Digital Pin 3 → 200Ω resistor → Ground

Figure 1. Dual LED Wiring Schematic — Arduino Uno R3 (Digital Pins 2 & 3)

## Software Architecture

The system implements a polled, round-robin Cyclic Executive task manager. Three tasks are defined and registered in a function pointer array: getSerialInput, blinkPin2, and blinkPin3. On every iteration of loop(), the Cyclic Executive iterates over the task array and calls each function in sequence. Each task uses millis()-based non-blocking logic and returns immediately if it has no work to do, ensuring no task blocks another.

Task Table:

Index 0: getSerialInput — checks for serial input and updates LED intervals

Index 1: blinkPin2 — checks and updates LED1 (Pin 2) blink state

Index 2: blinkPin3 — checks and updates LED2 (Pin 3) blink state

## Timing Operation

Timing operation is identical to PROJECT-ASYNC.

Ref: cmpe310.fall25.project4, DESIGN — Timing Operation section.

For an interval of X milliseconds:

LED ON for X/2 ms

LED OFF for X/2 ms

This cycle repeats continuously.

## Serial State Machine

The serial state machine logic is identical to PROJECT-ASYNC, now enclosed in getSerialInput().

Ref: cmpe310.fall25.project4, DESIGN — Serial State Machine section.

State WAIT_LED: Prompt for LED number (1 or 2).

State WAIT_INTERVAL: Prompt for interval in milliseconds.

After updating the selected LED interval, return to WAIT_LED.

## Source Code

## #include <Arduino.h>

## #define LED1_PIN 2

## #define LED2_PIN 3

## // ===== LED Control Structure =====

## struct LEDControl {

## int pin;

## unsigned long interval;

## unsigned long lastToggleTime;

## bool state;

## };

## LEDControl led1 = {LED1_PIN, 1000, 0, LOW};

## LEDControl led2 = {LED2_PIN, 1000, 0, LOW};

## // ===== Serial State Machine =====

## enum SerialState { WAIT_LED, WAIT_INTERVAL };

## SerialState currentState = WAIT_LED;

## int selectedLED = 0;

## // ===== Shared LED handler (non-blocking) =====

## void handleLED(LEDControl &led) {

## unsigned long now = millis();

## if (led.interval > 0) {

## if (now - led.lastToggleTime >= led.interval / 2) {

## led.state = !led.state;

## digitalWrite(led.pin, led.state);

## led.lastToggleTime = now;

## }

## }

## }

## // ===== Task functions =====

## void blinkPin2() { handleLED(led1); }

## void blinkPin3() { handleLED(led2); }

## void getSerialInput() {

## if (Serial.available() > 0) {

## int input = Serial.parseInt();

## if (currentState == WAIT_LED) {

## if (input == 1 || input == 2) {

## selectedLED = input;

## Serial.println("What interval (in msec)?");

## currentState = WAIT_INTERVAL;

## }

## }

## else if (currentState == WAIT_INTERVAL) {

## if (input > 0) {

## if (selectedLED == 1) led1.interval = input;

## else                  led2.interval = input;

## Serial.println("What LED? (1 or 2)");

## currentState = WAIT_LED;

## }

## }

## }

## }

## // ===== Cyclic Executive Task Table =====

## void (*tasks[])() = {getSerialInput, blinkPin2, blinkPin3};

## const int NUM_TASKS = sizeof(tasks) / sizeof(tasks[0]);

## // ===== Cycle Timing =====

## const unsigned long CYCLE_TIME = 10; // ms (adjust as needed)

## void setup() {

## pinMode(LED1_PIN, OUTPUT);

## pinMode(LED2_PIN, OUTPUT);

## Serial.begin(9600);

## while (!Serial);

## Serial.println("What LED? (1 or 2)");

## }

## // ===== Round-Robin Cyclic Executive =====

## void loop() {

## unsigned long cycleStart = millis();

## // Run all tasks once (round-robin)

## for (int i = 0; i < NUM_TASKS; i++) {

## tasks[i]();

## }

## // Wait until cycle time completes

## while (millis() - cycleStart < CYCLE_TIME) {

## // idle (do nothing)

## }

## }

# TESTING

## Retained Test Cases from PROJECT-ASYNC

Ref: cmpe310.fall25.project4, TESTING section.

Input: LED 2 → 600 ms

Expected: LED2 blinks 300ms ON / 300ms OFF

Input: LED 1 → 1600 ms

Expected: LED1 blinks 800ms ON / 800ms OFF. Verify other LED remains unaffected.

## Additional Test Cases (ECR 0.1)

T.9 — Cyclic Executive Registration:

Inspect source code to confirm that getSerialInput, blinkPin2, and blinkPin3 are all present in the tasks[] function pointer array.

T.10 — Round-Robin Dispatch:

Confirm that loop() iterates over all entries in tasks[] on every cycle with no task skipped or called out of order. Verify by code inspection and by observing that both LEDs blink and serial input is accepted simultaneously without interruption.

## Test Results

T.1 – Two LEDs verified connected and functional.

T.2 – Each LED independently configurable via serial input.

T.3 – Asynchronous operation confirmed; no LED pause during serial input.

T.4 – ON/OFF timing verified to equal interval/2.

T.5 – Blink rate stable over 30-second observation period.

T.6 – LED connections to Pins 2 and 3 confirmed.

T.7 – Required serial dialog sequence executes correctly.

T.8 – Successful compilation confirmed with Arduino IDE 2.3.3.

T.9 – All three tasks confirmed registered in the function pointer array.

T.10 – Round-robin dispatch confirmed by code inspection and live operation.

END OF DOCUMENT