# ASYNCHRONOUS DUAL LED EMBEDDED SYSTEM

Document name: Asynchronous Dual LED Embedded System

Document reference: .cmpe310.fall25.project4

Date of publication: February 24, 2026

LEAD ENGINEER: Oluwadamilola Kuforiji - University of Maryland Baltimore County

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA

MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

# HIGH-LEVEL DESCRIPTION

This project implements an asynchronous dual-LED control system on the Arduino Uno R3 (ATmega328P). The system allows a user to independently configure the blink interval of two LEDs via the Arduino IDE serial monitor. Each LED operates using non-blocking timing based on the millis() function, ensuring serial communication never interrupts LED blinking. The system satisfies all High-Level technical requirements and Educational Constraints defined for the PROJECT-ASYNC lab.

# DESCRIPTION

The system must control two LEDs connected to Digital Pins 2 and 3 of an Arduino Uno R3, allowing a user to independently set each LED's blink interval in milliseconds through the Arduino IDE serial monitor. The core challenge is ensuring that serial input processing does not block or interrupt the continuous blinking of either LED. This is solved using cooperative multitasking: LED timing is managed with millis()-based non-blocking checks, while a serial state machine processes user input asynchronously. The blink interval entered by the user defines the full period; each LED stays ON for interval/2 ms and OFF for interval/2 ms.

# REFERENCES

1. Arduino Reference – millis(): https://www.arduino.cc/reference/en/language/functions/time/millis/

2. ATmega328P Datasheet – Microchip Technology

3. Arduino IDE 2.3.3 Documentation: https://docs.arduino.cc/

# DEFINITIONS AND ABBREVIATIONS

ATmega328P – 8-bit AVR microcontroller used on the Arduino Uno R3

IDE – Integrated Development Environment

LED – Light Emitting Diode

millis() – Arduino function returning elapsed time in milliseconds since program start

Non-blocking – Code that does not halt program execution while waiting

Serial Monitor – Terminal interface in the Arduino IDE for serial communication

State Machine – Software construct that moves between defined states based on inputs

# CONVENTIONS

Must, shall, or will – Your design must satisfy the requirement.

May – Your design may satisfy the requirement but does not have to.

Informative – The intent of the following description is to make the requirement more understandable.

All customer requirements are prefixed with C.#

All high-level requirements are prefixed with HL.#

All testing/validation requirements are prefixed with T.#

# CUSTOMER REQUIREMENTS

C.1  The system must allow a user to independently control two LEDs.

C.1.1  The user must be able to set the blink interval of each LED separately.

C.2  The system must communicate with the user via the Arduino IDE serial monitor.

C.2.1  Serial communication must not interfere with LED blinking operation.

C.3  The system must run on a standard Arduino compatible development board.

# HIGH-LEVEL TECHNICAL REQUIREMENTS

HL.1  The System must use at least 2 LEDs.

HL.2  The System must use a standard Arduino compatible development board.

HL.3  The System must communicate via the Arduino IDE serial monitor.

HL.4  Serial communication must be asynchronous and must not affect LED blinking.

HL.5  The User must set blink interval in milliseconds.

HL.6  Blink rate must remain constant unless changed by the User.

Educational Constraints:

E.1  Code must compile using Arduino IDE 2.3.3 or later (gcc for ATmega328P).

E.2  All developer-created software must be hosted on GitHub.

E.2.1  Evaluator must be able to download, compile, and execute the software.

E.3  LEDs must be connected to Digital Pins 2 and 3.

E.4  System must perform required serial dialog sequence.

# TESTING AND VALIDATION REQUIREMENTS

T.1  Verify two LEDs are connected and operational.

T.2  Verify each LED can be independently configured.

T.3  Verify asynchronous operation (no LED pause during serial input).

T.4  Verify correct ON/OFF timing equals interval/2.

T.5  Verify blink rate stability over 30 seconds.

T.6  Verify LEDs are connected to Pins 2 and 3.

T.7  Verify required dialog executes correctly.

T.8  Verify successful compilation using Arduino IDE 2.3.3 or later.

# DESIGN

## Hardware Design:

Components:

- Arduino Uno R3 (ATmega328P)

- 2 LEDs

- 2 x 200Ω resistors

- USB cable for programming and power

Connection Details:

LED1 → Digital Pin 2 → 200Ω resistor → Ground

LED2 → Digital Pin 3 → 200Ω resistor → Ground

## Software Architecture:

The system uses cooperative multitasking. LED timing is controlled using millis() to avoid blocking delays. A serial state machine handles user interaction without interrupting LED blinking.

## Timing Operation:

For an interval of X milliseconds:

LED ON for X/2 ms

LED OFF for X/2 ms

This cycle repeats continuously.

## Serial State Machine:

State WAIT_LED: Prompt for LED number (1 or 2).

State WAIT_INTERVAL: Prompt for interval in milliseconds.

After updating the selected LED interval, return to WAIT_LED.

## Source Code

#include <Arduino.h>

#define LED1_PIN 2

#define LED2_PIN 3

// ===== LED Control Structure =====

struct LEDControl {

int pin;

unsigned long interval;

unsigned long lastToggleTime;

bool state;

};

LEDControl led1 = {LED1_PIN, 1000, 0, LOW};

LEDControl led2 = {LED2_PIN, 1000, 0, LOW};

// ===== Serial State Machine =====

enum SerialState { WAIT_LED, WAIT_INTERVAL };

SerialState currentState = WAIT_LED;

int selectedLED = 0;

// ===== Shared LED handler (non-blocking) =====

void handleLED(LEDControl &led) {

unsigned long now = millis();

if (led.interval > 0) {

if (now - led.lastToggleTime >= led.interval / 2) {

led.state = !led.state;

digitalWrite(led.pin, led.state);

led.lastToggleTime = now;

}

}

}

// ===== Task functions =====

void blinkPin2() { handleLED(led1); }

void blinkPin3() { handleLED(led2); }

void getSerialInput() {

if (Serial.available() > 0) {

int input = Serial.parseInt();

if (currentState == WAIT_LED) {

if (input == 1 || input == 2) {

selectedLED = input;

Serial.println("What interval (in msec)?");

currentState = WAIT_INTERVAL;

}

}

else if (currentState == WAIT_INTERVAL) {

if (input > 0) {

if (selectedLED == 1) led1.interval = input;

else                  led2.interval = input;

Serial.println("What LED? (1 or 2)");

currentState = WAIT_LED;

}

}

}

}

// ===== Cyclic Executive Task Table =====

void (*tasks[])() = {getSerialInput, blinkPin2, blinkPin3};

const int NUM_TASKS = sizeof(tasks) / sizeof(tasks[0]);

// ===== Cycle Timing =====

const unsigned long CYCLE_TIME = 10; // ms (adjust as needed)

void setup() {

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

Serial.begin(9600);

while (!Serial);

Serial.println("What LED? (1 or 2)");

}

# TESTING

## Test Execution and Results:

Example Test Cases:

Input: LED 2 → 600 ms

Expected: LED2 blinks 300ms ON / 300ms OFF

Input: LED 1 → 1600 ms

Expected: LED1 blinks 800ms ON / 800ms OFF. Verify other LED remains unaffected.

## Test Results:

T.1 – Two LEDs verified connected and operational.

T.2 – Each LED independently configurable via serial input.

T.3 – Asynchronous operation confirmed; no LED pause during serial input.

T.4 – ON/OFF timing verified to equal interval/2.

T.5 – Blink rate stable over 30-second observation period.

T.6 – LED connections to Pins 2 and 3 confirmed.

T.7 – Required serial dialog sequence executes correctly.

T.8 – Successful compilation confirmed with Arduino IDE 2.3.3.

END OF DOCUMENT