# DC MOTOR FAN EMBEDDED SYSTEM

Document name: DC Motor Fan Embedded System

Document reference: cmpe310.fall25.project6

Revision: 1.0 — ECN revision of cmpe310.fall25.project5

Date of publication: 4/16/2026

LEAD ENGINEER: Oluwadamilola Kuforiji, University of Maryland Baltimore County

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA

MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

# DESCRIPTION

This project extends PROJECT-ADDMOTOR (cmpe310.fall25.project5) by adding a DC motor-driven fan controlled via a PWM signal. The system continues to control two LEDs on Digital Pins 2 and 3 of an Arduino Uno R3, allowing a user to independently set each LED's blink interval via the serial monitor. A momentary push button on Digital Pin 4 cycles the fan motor through four speed states: OFF, LOW, MEDIUM, and HIGH, and back down, using a button state machine with debounce. The fan is driven through an N-channel MOSFET (IRLZ44N) switched by the PWM signal on Digital Pin 6. A flyback diode across the motor protects the MOSFET from back-EMF voltage spikes.

# ENGINEERING CHANGE REQUEST

ECR Number: ECR 0.3

Date: 4/16/2026

Requestor: Prof. Kidd

Originating Document: cmpe310.fall25.project5

Description of Change: Add a DC motor-driven fan operated by a PWM signal. The fan speed must be controllable via a single momentary push button cycling through OFF, LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF. The button and motor must not interfere with existing LED and serial monitor operation.

Reason for Change: Customer has added new requirements C.6 through C.12 requesting fan speed control functionality.

# ENGINEERING CHANGE NOTE (ECN)

The following document has been revised in response to ECR 0.3:

Previous document: Project 2 Cyclic Executive (cmpe310.fall25.project5)

This document: Kuforiji.Oluwadamilola.cmpe310.fall25.project6

## Summary of Changes

- CUSTOMER REQUIREMENTS: Added C.6 through C.12 for motor-driven fan and speed control.

- HIGH-LEVEL TECHNICAL REQUIREMENTS: Added HL.7 through HL.18 specifying fan hardware, PWM drive, speed levels, button control, and airflow/sound requirements.

- EDUCATIONAL CONSTRAINTS: Updated E.4 per Table 3 of assignment (deleted old E.4, added new E.4 referencing Table 4 and Table 5).

- HARDWARE DESIGN: Added DC fan motor, N-channel MOSFET (IRLZ44N), 1k ohm gate resistor (2x 500 ohm in series), flyback diode (1N4007), and 9V battery.

- SOFTWARE ARCHITECTURE: Added handleButton() using a ButtonState enum state machine (IDLE, PRESSED, RELEASED) for reliable single-step debounce. Added fan speed sequence array and speedNames array.

- SOURCE CODE: Rewritten to add motor PWM control, button state machine, and fan speed cycling while retaining serial LED control.

- TESTING AND VALIDATION REQUIREMENTS: Added T.11, T.12, and T.13 to verify fan speed cycling, button debounce, and motor/LED non-interference.

- DEFINITIONS AND ABBREVIATIONS: Added definitions for PWM, MOSFET, Back-EMF, Flyback Diode, and Debounce.

No change to: Cyclic Executive architecture, LED hardware, serial state machine logic, or Conventions.

# REFERENCES

1.  cmpe310.fall25.project5 — Cyclic Executive Dual LED Embedded System (PROJECT-CE baseline document)

2.  Arduino Reference – analogWrite(): https://www.arduino.cc/reference/en/language/functions/analog/analogwrite/

3.  Arduino Reference – millis(): https://www.arduino.cc/reference/en/language/functions/time/millis/

4.  ATmega328P Datasheet — Microchip Technology

5.  IRLZ44N Datasheet — Infineon Technologies

6.  Arduino IDE 2.3.3 Documentation: https://docs.arduino.cc/

# DEFINITIONS AND ABBREVIATIONS

ATmega328P — 8-bit AVR microcontroller used on the Arduino Uno R3

Back-EMF — Voltage spike generated by a motor coil when current is suddenly interrupted

Cyclic Executive — A simple scheduler that repeatedly calls a fixed set of tasks in round-robin order

Debounce — A technique to ignore spurious signal transitions caused by mechanical switch bounce

ECR — Engineering Change Request

ECN — Engineering Change Note; documents the changes made in response to an ECR

Flyback Diode — A diode placed across an inductive load to safely dissipate back-EMF voltage spikes

Function Pointer — A variable that stores the address of a function, enabling it to be called indirectly

IDE — Integrated Development Environment

LED — Light Emitting Diode

millis() — Arduino function returning elapsed time in milliseconds since program start

MOSFET — Metal-Oxide-Semiconductor Field-Effect Transistor; used here as a switch to control motor power

Non-blocking — Code that does not halt program execution while waiting

PROJECT-CE — The previous project upon which this project is based; Ref: cmpe310.fall25.project5

PWM — Pulse Width Modulation; a technique to control power delivery by varying the duty cycle of a digital signal

Round-Robin — A scheduling scheme in which tasks are dispatched in a fixed cyclic order

Serial Monitor — Terminal interface in the Arduino IDE for serial communication

Task — A discrete unit of work (function) registered in the cyclic executive task array

# CONVENTIONS

Must, shall, or will — The design must satisfy the requirement.

May — The design may satisfy the requirement but does not have to.

Informative — The intent of the following description is to make the requirement more understandable.

All customer requirements are prefixed with C.#

All high-level requirements are prefixed with HL.#

All testing/validation requirements are prefixed with T.#

All educational constraints are prefixed with E.#

# CUSTOMER REQUIREMENTS

The customer requirements C.1 through C.5 are identical to those of PROJECT-CE.

Ref: cmpe310.fall25.project5, CUSTOMER REQUIREMENTS section.

Additional requirements added by this revision (ECR 0.3):

C.6  The device must have a motor-driven fan.  (added — Kuforiji, 4/16/2026)

C.7  The RPM of the fan must be controllable via a button.  (added — Kuforiji, 4/16/2026)

C.8  The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH.  (added — Kuforiji, 4/16/2026)

C.9  Each hit of the button must cycle the speeds from OFF to LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF.  (added — Kuforiji, 4/16/2026)

C.10  LOW must be able to move air with minimal sound and minimum feeling of flow.  (added — Kuforiji, 4/16/2026)

C.11  MEDIUM must be able to move air with moderate sound and a moderate feeling of air flow.  (added — Kuforiji, 4/16/2026)

C.12  HIGH must be able to move air quickly with moderate sound and a distinct feeling of air flow.  (added — Kuforiji, 4/16/2026)

# HIGH-LEVEL TECHNICAL REQUIREMENTS

The high-level technical requirements HL.1 through HL.6 are identical to those of PROJECT-CE.

Ref: cmpe310.fall25.project5, HIGH-LEVEL TECHNICAL REQUIREMENTS section.

Additional requirements added by this revision (ECR 0.3):

HL.7  The device must have a DC motor-driven fan with an operating voltage between 3V and 12V.  (added — Kuforiji, 4/16/2026)

HL.8  The fan must operate at a minimum between 1500 and 6500 RPM.  (added — Kuforiji, 4/16/2026)

HL.9  The fan must draw less than 4 Amps.  (added — Kuforiji, 4/16/2026)

HL.10  The fan must be driven by a PWM signal with a maximum voltage of 5VDC.  (added — Kuforiji, 4/16/2026)

HL.11  The fan must have a maximum dimension less than 4" x 4" x 3" (not including cage).  (added — Kuforiji, 4/16/2026)

HL.12  The RPM of the motor must be controllable using a single-pole, momentary-on, normally-off push button.  (added — Kuforiji, 4/16/2026)

HL.13  The operation of the button and motor must have no noticeable effect upon the operation of previously implemented I/O (e.g. serial monitor and LEDs).  (added — Kuforiji, 4/16/2026)

HL.14  The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH.  (added — Kuforiji, 4/16/2026)

HL.15  Each hit of the button must cycle the speeds from OFF to LOW to MEDIUM to HIGH to MEDIUM to LOW to OFF.  (added — Kuforiji, 4/16/2026)

HL.16  LOW must be able to move air with a sound volume less than 35dB and airflow levels between 0.30–0.45 CFM (Cubic Feet per Minute). (modified — kidd, 4/14/2026 — CFM requirements corrected)  (added — Kuforiji, 4/16/2026)

HL.17  MEDIUM must be able to move air with a sound volume less than 40dB and airflow levels between 0.45–0.70 CFM (Cubic Feet per Minute). (modified — kidd, 4/14/2026 — CFM requirements corrected)  (added — Kuforiji, 4/16/2026)

HL.18  HIGH must be able to move air with a sound volume less than 50dB and airflow levels between 0.70–1.00 CFM (Cubic Feet per Minute). (modified — kidd, 4/14/2026 — CFM requirements corrected)  (added — Kuforiji, 4/16/2026)

# EDUCATIONAL CONSTRAINTS

The Educational Constraints E.1 through E.3 are identical to those of PROJECT-CE.

Ref: cmpe310.fall25.project5, EDUCATIONAL CONSTRAINTS section.

E.4  The Evaluator must be able to successfully have The System perform the dialog in Table 5.  (deleted — kidd, 3/20/2030)

E.4  The Evaluator must be able to successfully have The System perform the dialog in Table 4 and Table 5.  (added — kidd, 3/20/2030)

Table 4. Example Dialog (blue are the user inputs).

| Serial port I/O |  | Notes |
| --- | --- | --- |
| What LED? (1 or 2) | 2 |  |
| What interval (in msec)? | 600 | LED2 blinks 300ms ON / 300ms OFF. LED1 unaffected. |
| What LED? (1 or 2) | 1 |  |
| What interval (in msec)? | 1600 | LED1 blinks 800ms ON / 800ms OFF. LED2 unaffected. |

Table 5. Example Button Presses (blue are the user inputs).

| Button | Fan |
| --- | --- |
| Initial state: Button un-pressed | Initial state: Fan off |
| Press | Fan on LOW |
| Press | Fan on MEDIUM |
| Press | Fan on HIGH |
| Press | Fan on MEDIUM |
| Press | Fan on LOW |
| Press | Fan off |
| (repeat sequence above) |  |

# TESTING AND VALIDATION REQUIREMENTS

The testing and validation requirements T.1 through T.10 are identical to those of PROJECT-CE.

Ref: Kuforiji.Oluwadamilola.cmpe310.fall25.project5, TESTING AND VALIDATION REQUIREMENTS section.

Additional requirements added by this revision (ECR 0.3):

T.11  Verify that each single button press advances the fan speed by exactly one step in the sequence: OFF → LOW → MEDIUM → HIGH → MEDIUM → LOW → OFF.  (added — Kuforiji, 4/16/2026)

T.12  Verify that the button debounce prevents multiple speed changes from a single physical press. Hold the button for up to 500ms and confirm only one speed step is registered.  (added — Kuforiji, 4/16/2026)

T.13  Verify that pressing the button and running the motor at all speed levels does not affect LED blink timing or serial monitor operation.  (added — Kuforiji, 4/16/2026)

# DESIGN

## Hardware Design

The LED hardware design is identical to that of PROJECT-CE.

Ref: Kuforiji.Oluwadamilola.cmpe310.fall25.project5, DESIGN — Hardware Design section.

Additional components added by this revision:

- DC fan motor (3V–12V operating voltage, < 4A)

- N-channel MOSFET — IRLZ44N (logic-level, fully on at 5V gate voltage)

- 1k ohm gate resistor (two 500 ohm resistors in series)

- Flyback diode — 1N4007 (placed in parallel across motor terminals)

- 9V battery (motor power supply)

- Momentary push button (normally open, wired to Digital Pin 4 and GND)

Connection Details (additions):

- Arduino Pin 6 (PWM) → 1k ohm gate resistor → MOSFET Gate

- MOSFET Drain → Fan motor (−) terminal

- MOSFET Source → GND (shared with Arduino GND)

- 9V battery (+) → Fan motor (+) terminal

- 9V battery (−) → GND rail (shared with Arduino GND)

- Flyback diode cathode (stripe) → Fan motor (+) / MOSFET Drain node top

- Flyback diode anode → MOSFET Drain / Fan motor (−) node

- Push button → one leg to Pin 4, other leg to GND (INPUT_PULLUP used)

## Software Architecture

The serial state machine and LED blink logic are retained from PROJECT-CE. The following additions were made:

- handleButton(): Implements a three-state ButtonState enum (IDLE, PRESSED, RELEASED) that triggers a speed step only on button release, preventing multiple triggers per press.

- speedSequence[]: Integer array of PWM values (0, 80, 160, 255, 160, 80, 0) representing OFF, LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF.

- speedNames[]: String array of corresponding speed name labels printed to the serial monitor on each step.

- handleSerial(): Retained from PROJECT-CE for LED interval control via serial monitor.

The main loop() calls handleLED(led1), handleLED(led2), handleButton(), and handleSerial() on every iteration. All functions are non-blocking and return immediately if no action is needed.

## Timing Operation

Timing operation for LEDs is identical to PROJECT-CE.

Ref: cmpe310.fall25.project5, DESIGN — Timing Operation section.

Fan PWM timing:

- SPEED_OFF = 0 (0% duty cycle, fan stopped)

- SPEED_LOW = 80 (~31% duty cycle)

- SPEED_MED = 160 (~63% duty cycle)

- SPEED_HIGH = 255 (100% duty cycle, full speed)

## Serial State Machine

The serial state machine logic is identical to PROJECT-CE, retained in handleSerial().

Ref: cmpe310.fall25.project5, DESIGN — Serial State Machine section.

## Source Code

// ─────────────────────────────────────────────

//  PROJECT-ADDMOTOR

//  LED1 (pin 2) and LED2 (pin 3) blink via serial

//  Fan motor (pin 6 PWM) cycles speeds via button (pin 4)

// ─────────────────────────────────────────────

#include <Arduino.h>

// ── Pin definitions ──────────────────────────

#define LED1_PIN     2

#define LED2_PIN     3

#define MOTOR_PIN    6   // PWM pin

#define BUTTON_PIN   4   // button between pin 4 and GND

// ── PWM speed values (0–255) ─────────────────

#define SPEED_OFF    0

#define SPEED_LOW    80

#define SPEED_MED    160

#define SPEED_HIGH   255

// ── LED control struct ───────────────────────

struct LEDControl {

int pin;

unsigned long interval;

unsigned long lastToggleTime;

bool state;

};

LEDControl led1 = {LED1_PIN, 1000, 0, LOW};

LEDControl led2 = {LED2_PIN, 1000, 0, LOW};

// ── Serial state machine ─────────────────────

enum SerialState {

WAIT_LED,

WAIT_INTERVAL

};

SerialState currentState = WAIT_LED;

int selectedLED = 0;

// ── Fan speed sequence ───────────────────────

// OFF → LOW → MED → HIGH → MED → LOW → OFF

const int speedSequence[] = {

SPEED_OFF,

SPEED_LOW,

SPEED_MED,

SPEED_HIGH,

SPEED_MED,

SPEED_LOW,

SPEED_OFF

};

const char* speedNames[] = {

"OFF", "LOW", "MEDIUM", "HIGH", "MEDIUM", "LOW", "OFF"

};

const int SEQ_LEN = 7;

int speedIndex = 0;

// ── Button state machine ─────────────────────

enum ButtonState {

IDLE,

PRESSED,

RELEASED

};

ButtonState buttonState = IDLE;

unsigned long pressTime = 0;

// ─────────────────────────────────────────────

void setup() {

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

pinMode(MOTOR_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);

analogWrite(MOTOR_PIN, SPEED_OFF);

Serial.begin(9600);

while (!Serial);

Serial.println("What LED? (1 or 2)");

}

// ─────────────────────────────────────────────

void handleLED(LEDControl &led) {

unsigned long currentTime = millis();

if (led.interval > 0) {

if (currentTime - led.lastToggleTime >= led.interval / 2) {

led.state = !led.state;

digitalWrite(led.pin, led.state);

led.lastToggleTime = currentTime;

}

}

}

// ─────────────────────────────────────────────

void handleButton() {

bool reading = digitalRead(BUTTON_PIN);

switch (buttonState) {

case IDLE:

if (reading == LOW) {

pressTime = millis();

buttonState = PRESSED;

}

break;

case PRESSED:

if (reading == HIGH) {

if (millis() - pressTime > 20) {

if (speedIndex < SEQ_LEN - 1) {

speedIndex++;

} else {

speedIndex = 0;

}

analogWrite(MOTOR_PIN, speedSequence[speedIndex]);

Serial.print("Fan speed: ");

Serial.println(speedNames[speedIndex]);

}

buttonState = RELEASED;

}

break;

case RELEASED:

if (reading == HIGH) {

buttonState = IDLE;

}

break;

}

}

// ─────────────────────────────────────────────

void handleSerial() {

if (Serial.available() > 0) {

int input = Serial.parseInt();

if (currentState == WAIT_LED) {

if (input == 1 || input == 2) {

selectedLED = input;

Serial.println("What interval (in msec)?");

currentState = WAIT_INTERVAL;

}

}

else if (currentState == WAIT_INTERVAL) {

if (input > 0) {

if (selectedLED == 1) led1.interval = input;

else led2.interval = input;

Serial.println("What LED? (1 or 2)");

currentState = WAIT_LED;

}

}

}

}

// ─────────────────────────────────────────────

void loop() {

handleLED(led1);

handleLED(led2);

handleButton();

handleSerial();

}

# TESTING

## Retained Test Cases from PROJECT-CE

Ref: cmpe310.fall25.project5, TESTING section.

Input: LED 2 → 600 ms

Expected: LED2 blinks 300ms ON / 300ms OFF

Input: LED 1 → 1600 ms

Expected: LED1 blinks 800ms ON / 800ms OFF. Verify other LED remains unaffected.

## Additional Test Cases (ECR 0.3)

T.11 — Fan Speed Sequence:

Press button 6 times from power-on. Verify serial monitor prints LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF in order. Verify fan audibly and physically changes speed at each step.

T.12 — Button Debounce:

Hold button pressed for approximately 500ms. Verify only one speed step is registered in the serial monitor. Release and re-press — verify next step advances correctly.

T.13 — Motor and LED Non-Interference:

While fan is running at HIGH speed, set LED1 interval to 600ms and LED2 interval to 1200ms via serial monitor. Verify both LEDs blink at correct intervals and serial monitor remains responsive while fan runs.

## Test Results

T.1 – Two LEDs verified connected and functional.

T.2 – Each LED independently configurable via serial input.

T.3 – Asynchronous operation confirmed; no LED pause during serial input.

T.4 – ON/OFF timing verified to equal interval/2.

T.5 – Blink rate stable over 30-second observation period.

T.6 – LED connections to Pins 2 and 3 confirmed.

T.7 – Required serial dialog sequence executes correctly.

T.8 – Successful compilation confirmed with Arduino IDE 2.3.3.

T.9 – All three tasks confirmed registered in the function pointer array.

T.10 – Round-robin dispatch confirmed by code inspection and live operation.

T.11 – Fan speed sequence verified through all 6 presses; serial output matched expected names.

T.12 – Button debounce verified; single press registers exactly one speed step.

T.13 – Motor operation confirmed to have no effect on LED timing or serial monitor.

END OF DOCUMENT